Pinger

Version 1.4

http://www.ibt.com/~estrella/index.html

Overview -

The pinger program is designed to ping a user provided list of systems and warn you if a system goes down.  The system has been designed to be easy to use and perform with the minimal of requirements.  I personally use it to monitor 183 addresses (routers, router interfaces, hubs, servers, etc) from my regular NT workstation system with no impact on its performance.

Information about the latest version of the program is available at http://www.ibt.com/~estrella/index.html

New Features in this version -

- Changes to the current documentation appear in red.

- Traceroute utility.

- Play WAV file if a system goes down.

- Ability to clear history screen.

- Fixed bug that allowed polling to be set at zero tries.

Key Features -

- Able to automatically maintain HTML pages showing systems that are currently down and also the history of systems that came down and back up.

- Ability to save outages into a user defined file.

- Ability to execute a BAT file each time a system goes down.

- Abuility to play a WAV file when a system goes down.

- TraceRoute utility fully integrated.

- Fully threaded application for quicker response.

- Low resource usage.

- System designed to run in background at all times.

- Runs on the system icon tray for ease of use.

- Full use of the Registry to save options.

Requirements -

- Windows 95 or Windows NT 4.0.

- Microsoft TCP/IP stack.

How to install the program -

- Unzip the files into a directory.

- Run the setup to install the program.

- To uninstall, execute the uninstall routine through the normal windows Program Add/Remove option.

How to run the program -

- Execute the pinger from the start menu.

- We recommend that you place this file in the startup folder so that it start automatically.

- To see the detail screen showing the status of the systems being monitored, double click on the icon for the program at the icon tray area (usually lower right corner - next to clock).

- During program execution, if an error occur, the error screen will pop-up so you can see.

- You can enable an option to have the display screen pop-up if changes occur.

- As system fail and then come back up, they will be placed on the History pane of the detail screen.  If you need to clear this screen, just right click on it and select the clear screen option.

- To traceroute, you can select a system from the detail screen, right click and you will see the traceroute to it.  You can also just right click on the history or down pane without selecting a system.  This way, a popup will ask for the address of the system to traceroute to.

How to configure the program -

- To get to the configuration screen, place cursor under the icon for the program on the Icon Tray area (usually lower right corner - next to clock).  Right click to see the options menu.

- Select configure option.

- Key options to note are:

- System to Monitor -  Click on new and then enter the ip address (as name or numeric notation) of the system to be monitor.  Obviously repeat as needed.

- Polling:  Options to determine how often to check systems.  Increase the Retries before notifying down and Seconds to wait for response if you get too many false alarms.

- Logging:  Enable this option to save the record of systems going down.  Note that a record is comma delimited and will only get written after the system comes back up.

- Notification:  Options for dealing with notifying operator when error occur.  Note that if you select the option to Continue to beep until acknowledged once an error occurs the system will beep every 5 seconds.  To acknowledge it, a new button will appear on the Details screen (lower left corner).  If you select the option to execute a BAT file when an error occur, you should note that the system doesn't check to see if the file is executed successfully.  Also, the BAT file is called with the system being down as the only option. (See demo.bat as an example of how to utilize this).  You also have the ability to play a WAV file when a system goes down.

- HTML:  Enable the creation of HTML files automatically.  With this option you can have the system automatically create HTML files, and place them on a directory where the WWW server can serve them (ex. Do a net use to the file system where the server is at).  Options Unavailable and History filename are the file names of the HTML files to be created by the program.  The header and footer filename are file names for a header and footer HTML file that allows you to customize the presentation of the files with backgrounds, Company logos, etc.  See files header.htm and footer.htm for examples.

Notes -

- If the system fails to work, ensure that TCP/IP is working OK by using the DOS ping command.

- Currently only 200 systems can be monitored.  This will be dramatically improved in upcoming versions.  If you need to monitor additional system, let me know and I will create a temporary release just fixing this.

- Once a DNS fails for an address, the system will stop monitoring it.

- Every time you exit the configuration screen, those options are loaded into the program.

History -

- Version 1.4 released on November 22, 1996.

- Changes to the current documentation appear in red.

- Traceroute utility.

- Play WAV file if a system goes down.

- Ability to clear the history screen.

- Fixed bug that allowed polling to be set at zero tries.

- Version 1.3 released on October 14, 1996.

- Support for HTML.

- Support for saving records to a file.

- Ability to execute a BAT file when a system fails

- Continuous notification until acknowledge by operator.

- Improved read-me file.

- Version 1.2 released on October 1, 1996.

- Ensures only one copy of program runs.

- Improved polling logic to reduce false alarms.

- Configuration changes take effect in session.

- Version 1.1 released on September 25, 1996.

- Fixes a problem in which some errors like host unreachable are not properly detected.

- Version 1.0 Released on September 21, 1996

Product Direction -

NOTE: Timeframes will depend on current work situation.  My current activities have been low so I have had time to work on this after work and during lunch hour.  Also, as I get feedback from users, I will try to incorporate those into the software (For example, the active notification  was a user request).

My current hopes are:

- The next version will be an incredible major upgrade.  It will include:

- Complete resign of the base software and logic.

- Changes to the user interface.

- Ability to have many other fields when describing objects to be monitored (ex location, system type, etc)

- Ability to have different Polling for different devices.

- The next version will be a major upgrade.  It will include:

- Client-Server approach.  Client will be thin and will allow clients in the future to be written in Java for WWW access.  Will allow many users to be connected to the one system through TCP-IP thus decreasing network polling.

- Security through user-id and passwords.

- The next version will be a major upgrade.  It will include:

- Ability to monitor not only IP systems by ping, but also SNMP.

- Ability to have different Notifications for different devices.

- The following version will be a major upgrade.  It will include:

- Ability to monitor non-IP devices such as servers running netbeui or Novell.

- Ability to do notification through paging.

- Ability to monitor links off devices (FDDI, ethernet, PPP and frame-relay).

- Ability to send e-mail when systems goes down.

- Ability to send daily reports through email (ex. Last 24 hour activity).

- The following version will be a major upgrade.  It will include:

- Ability to monitor user selected SNMP attributes.

- Ability to monitor applications such as TELNET, SMTP, WWW, etc.

Additional features that I would like to include but have not determine yet how to fit them -

- Run as an NT service.

- Ability to define consistent parameters to be monitored across devices (ex utilization, memory, etc).

- Generic application to monitor links and devices through SNMP.

- Ability to save to a DB system such as access or an ODBC system automatically.

- Clients for accessing system from UNIX and Java.

Freeware vs Shareware vs Costware -

I will continue to provide the base product as a freeware product.  However, some of my plans call for features that require me to purchase components (SNMP is the most obvious).  As such, I am currently trying to figure out the best way to address this issue.

My current hope is that I will continue to offer the product as freeware (ping checks, HTML, etc), and for more advance features for which I need to buy software (paging, SNMP,. etc) I will charge a small fee.

I have not made a decision here and hope to get feedback from people as to what they think.

Last word -

Enjoy and hope this is a useful product for you.